# Technical Note

## Removing and Installing the IJ-6000 USB Printer Drivers

## In Windows XP

Summary:

This document describes how to install and remove USB printing support for Addmaster IJ-6000 printers. Windows 98/ME/2K/XP maintains data-bases of previously loaded device drivers. This feature complicates the removal of device drivers for software developers who may wish to test various versions of a particular device driver.

Addmaster supports two different device drivers for its IJ-6000 USB attached printers.

This document will present procedures allowing a developer to switch between the two drivers during the test & development process.

By:		Engineering Department

Addmaster Corporation

Date:		July 23, 2002

Revision:	2.0

# 1. 	Introduction

1.1  Overview & Use

The IJ-6000 series of Ink-Jet Transaction Printers are modern, high-reliability transaction printers designed for use in financial, POS and other applications.  The printers are available with serial, parallel, or USB interfaces.

Addmaster provides two different types of drivers for the USB attached printers:

| Bi-directional | User communicates via bi-directional serial data stream with printer. Status back is supported. Programming is required. |
| --- | --- |
| Mini-driver | Typical Windows printer driver. Status back is not supported. Programming is optional. |

This document will describe how to switch back and forth between these two drivers on one Windows based computer.

An end-user would never perform such a procedure. The software developer would typically select one method for USB support and the end-user would load the supporting driver only once. However, while the software developer is evaluating the different drivers, a need to switch among these drivers may occur. This process of switching drivers is complicated by a feature of Windows, where Windows maintains a data-base of previously loaded device drivers. This information is kept for the purposes of aiding end-users by automating the device driver selection process. This feature, however, makes more difficult the task of evaluating different drivers each supporting the same device.

1.2   Assumptions

It is not assumed that one of the two USB drivers is already loaded onto the computer. If this is a first installation, begin at section 2 to install the mini-driver, and section 3 to install the bi-directional driver. Begin at section 4 to replace the mini-driver with the bi-directional driver, and begin at section 5 to replace the bi-directional driver with the mini-driver.

Other available documentation concerning the Addmaster IJ-6000 series printers is:

- IJ-6000 Ink-jet Transaction Printer Specification

- IJ-6080 Ink-jet Transaction Printer User’s Guide

- IJ-6000 Ink-jet Transaction Printer Flash Programming Guide.

- New Installation of IJ-6000 with Mini-Driver

Follow the step-by-step instructions in this Section when installing the software driver for Windows XP machines. Installation consists of these steps:

- Preparing the IJ-6000 printer and the computer for installation.

- Installing Microsoft’s USB printer driver and Addmaster’s low level USB driver.

- Installing Addmaster’s IJ-6000 mini-driver.

## Step 1 – Preparation

Prepare for installation by collecting software and documentation:

- Have a copy of these instructions handy for reference.

- Have the installation diskette containing the USB and printer drivers handy.

- Turn-off the IJ-6080 printer.

- Connect USB cable to computer.

- Connect USB cable to IJ-6080 printer.

## Step 2 – Installing Microsoft’s USB Printer Driver

The next step involves loading the device drivers responsible for communications on the USB port with the IJ-6000 printer.  This can be accomplished only with an IJ-6000 USB printer on hand and cannot be done by proxy. Each step must be followed carefully.

You should have an unpowered IJ-6000 cabled to your PC at this point (see Preparation).

Windows XP will now attempt to install the new drivers.  Follow these steps:

- Power-on the IJ-6000 printer. Windows XP will detect the printer and initiate Plug and Play, displaying the “Found New Hardware” message for “Unknown Device.”

- When WinXP displays the “Add New Hardware Wizard,” select “Install from a List or Specific Location,” and click “Next.”

- Insert the IJ-6000 Driver Diskette in drive A.

- When asked, choose “Search for the best driver” and select “Include this Location in Search”, enter “A:\” as the location, and click “Next.”

- One of three situations can occur.

  - Windows XP will find “A:\add_b.inf” as the best driver, and begin copying files from the diskette. You may not be notified that Windows XP has found, and is using, “A:\add_b.inf”, but you will be able to observe activity on drive A.

  - If Windows XP informs it can not find a better driver for the device, either the wrong diskette has been inserted, or the old drivers have not been correctly removed. Click “Back”, check the correct diskette is inserted, and click “Next.” If the problem is not fixed call the factory for assistance.

  - If Windows XP asks you to select from a list of drivers. This situation can arise if a driver has been installed on the machine at some time for a USB device that used the same Cypress USB chip as in the Addmaster IJ-6000 printer. If you started this procedure by removing previous IJ-6000 drivers, select the “A:\add_b.inf “ entry and click “Next.” If “A:\add_b.inf” is not in the list, or if you did not begin this procedure by removing old IJ-6000 drivers, call the factory for assistance.

- Windows XP will display a copying files message. If Windows XP warns that the driver has not passed logo testing to verify its compatibility with windows XP, respond by clicking “Continue Anyway.” After files are copied and the driver is installed, the Hardware Update Wizard will inform that it has finished installing the software for “CYPRESS EZ-USB (2131Q/2131S/2135S) – EEPROM missing.” Click “Finish” to close the wizard.

From this point on, the process should be automatic. After communicating with the driver Windows has installed, the printer will momentarily disconnect from the USB bus and then reconnect, asking Windows for a second driver to be loaded. This second driver should be loaded with none to minimal user interaction.

- The Found New Hardware Wizard may briefly display a message to inform that it is installing software for “SERIAL #34567” and then for “USB Printing Support.”

- Follow any instructions as required. Specifically, specify location “A:\” to search for a driver, and click “Continue Anyway” if challenged that the driver has not passed logo testing.

- Open the Device Manager and open the “Universal Serial Bus Controller” entry. If you see an “USB Printing Support” entry, windows has completed installing the second driver.

The driver loaded will be the Microsoft USB Printer Driver and requires a mini-driver to be installed to perform printing. This is performed during the next step.

## Step 3 – Installing IJ-6000 Mini-Driver

1.	Open the Printers and Faxes folder. Click {Start, Printers and Faxes}.

2.	Double click “Add Printer.” Windows will display the Add Printer Wizard.

3.	Choose “Local Printer” and click “Next.”

- Select from the list of ports. At the bottom will be an entry such as “USBnnn (Virtual printer port for USB)” entries, where nnn are numbers from 000 and 999. Typically only one entry will appear, but if previous instances of the driver were not completely removed, there may be more entries, and some trial and error may be required later to determine the appropriate selection has been made. For now, pick a USB port and click “Next,” to proceed to select the printer model

- “Have Disk” when prompted for printer make and model. The “Install from Disk” window will appear.

- Insert driver diskette into A: Use “Browse” button to select location of driver as A: Click “OK.”

- Windows will display the name of the printer type for which installation files were found. Click “Next.”

- Select “Replace existing driver” if asked and click “Next.”

- Choose a printer name, say “IJ-6000.”

- Select “Use as default” and click “Next.”

- Select “Print test page.” and click “Next.”

- Windows will now display the “Completing the Add Printer Wizard” window. Click “Finish” to close the window.

- The process will complete automatically from here. If Windows warns, “The software you are installing for this hardware has not passed Windows Logo testing to verify its compatibility with Windows XP” click “Continue Anyway.”

- Windows will copy files from the driver diskette and inform that it has sent the test page to the printer.

- Proceed to Step 4, Check Out & Test of Mini-Driver.

## Step 4 – Check Out & Test of Mini-Driver

If you skipped steps 3 above,

- Right click on the IJ-6000 printer icon and select “Properties.”

- Click “Print Test Page.”

If you performed step 3 above, continue here.

- If you get a message reporting an error  “Problem printing to port,” then click “OK” and proceed.

- You will be asked “Did the test page print properly?”  Regardless of whether or not it did, click “YES”. This will save time.

- If the test page actually did print properly, you are done.  If the test page failed to print properly, first check to see if both of the printer’s green lights are alight. If they are, it is waiting for a cut-form to print the test page on. Insert a cut-form and see if this fixes the problem. If it does, you are done; otherwise, continue.

- Select the “Details” tab.

- Go to the field “Print to the following port” and click the DOWN-ARROW key.

- Select another USBxxx Virtual Printer Port entry.

- Click “Apply” and then click “OK”.

- Repeat the Check Out & Test of Mini-driver process beginning at step 1.

- New Installation of Bi-directional Driver

Follow the step-by-step instructions in this Section when installing the software driver for Windows XP machines. Installation consists of these steps:

1.	Preparing the IJ-6000 printer and the computer for installation.

- Installing Microsoft’s USB printer driver and Addmaster’s low-level USB driver.

- Installing Addmaster’s IJ-6000 mini-driver

- Replacing the mini-driver with the bi-directional driver

## Step 1 – Preparation

Prepare for installation by collecting software and documentation:

1.	Have a copy of these instructions handy for reference.

2.	Have the installation diskette containing the USB and printer drivers handy.

3.	Turn-off the IJ-6080 printer.

4.	Connect USB cable to computer.

- Connect USB cable to IJ-6080 printer.

## Step 2 – Installing Microsoft’s USB Printer Drivers

Perform Step 2 from Section 2, New Installation of IJ6000 with Mini-Driver.

## Step 3 – Installing IJ-6000 Mini-Driver

Perform Step 3 from Section 2, New Installation of IJ6000 with Mini-Driver.

## Step 4 – Check Out & Test of Mini-Driver

Perform Step 4 from Section 2, New Installation of IJ6000 with Mini-Driver.

## Step 5 – Installing and Switching to the Bi-directional Driver

It is necessary to switch to the bi-directional driver now. A user program is provided to perform the switch. To use, run the program:

A:\ChooseIj6Driver.exe

Follow these steps:

1.	Select “Bidirectional Driver” from the list and click “Apply Choice.”

- When message “Done - Turn Printer OFF/ON to Take Effect,” appears, do so.

- If a prior bi-directional driver was installed, Windows XP will have a memory of this. Therefore, open the Device Manager and open the “Universal Serial Bus Controller” entry. If the “USB Printing Support” entry has a yellow “!” (exclamation mark) beside it, right click the entry or click “Action” from the menus, and then click “Uninstall.” Otherwise, proceed to Step 5 for testing.

- Cycle printer power to activate “Found New Hardware” wizard.

- The wizard will ask you to insert the installation floppy. Insert the IJ-6000 Driver Diskette in drive A:\.

- Select “Install the Software Automatically” and click “Next”.

- Windows will access the diskette and may warn “The software you are installing for this hardware has not passed Windows Logo testing to verify its compatibility with Windows XP.” Click “Continue Anyway.”

- Wait while Windows copies files from the diskette and click “Finish” when it is done to close the wizard.

- Down in the bottom right hand corner of the screen you may catch a status message that the hardware is installed and ready to use.

## Step 6 – Check Out & Test of Bi-directional Driver

A test program is provided to test the connection. To use, run the program:

A:\IJ6000_Test.exe

This program allows for testing the journal, validation, and status back features of the printer.

If the test program fails to print properly check these items:

- Check printer DIP switches and ensure they appear in the default state.

- Make sure the IJ-6000 printer is not in its self-test mode. Turn it off and on. Wait for USB drivers to automatically reset. Try the test over again.

4.	Replacing Mini-Driver with Bi-directional Driver

Follow the step-by-step instructions in this Section when installing the software driver for Windows XP machines. Installation consists of these steps:

1.	Preparing the IJ-6000 printer and the computer for installation.

2.	Removing any older IJ-6000 series printer drivers and any older Addmaster USB drivers.

3.	Installing Addmaster’s USB driver and Microsoft’s USB Printer Driver.

4.	Installing Addmaster’s IJ-6000 mini-driver. (optional)

5.	Installing the USB bi-directional printer driver.

## Step 1 – Preparation

Prepare for installation by collecting software and documentation:

1.	Have a copy of these instructions handy for reference.

- Have the installation diskette containing the USB and printer drivers handy.

- Turn-off the IJ-6080 printer.

- Connect USB cable to computer.

- Connect USB cable to IJ-6080 printer.

## Step 2 – Removing Old USB & Print Drivers

If you are certain that you have the latest IJ-6000 printer drivers (mini-driver and bi-directional driver) installed, skip ahead to step 4. Otherwise, continue by removing and replacing existing drivers first.

Any existing Windows USB printing support for the IJ-6000 should be removed. The IJ-6000 mini-driver can remain.

To remove old USB drivers:

- Connect the IJ-6000 Printer and turn it on.

- Optionally delete any IJ-6000 printers from {Start, Printers and Faxes}. Skip this step if you wish to retain the mini-driver.

- Open the Device Manager, click {Start, Control Panel, Printers and Other Hardware (switch to “Classic View”, if necessary), System, Hardware, Device Manager}.

- Open the “Universal serial bus controller” entry.

- Right click on “USB Printing Support,” or click “Action” on the top menu.

- Click the “Uninstall” option from the list menu to remove the device, and click “OK” to confirm.

- Insert “IJ-6000 Driver Diskette” into floppy drive A:

- Run program: “A:\CLEAN_XP.exe” (removes ezusb.sys, adm_ldr.sys)

- Power-off then Power-on the IJ-6000 printer.

- In the Device Manager, select the “Universal serial bus controller” entry.

- Open the “Universal serial bus controller” entry.

- Right click on the “Cypress EZ-USB( 2131Q/2131S/2135S) - EEPROM missing” entry.

- Click the “Properties” option from the list menu.

- Click the “Driver” tab and then click “Update Driver.”

- Windows XP will display the “Hardware Update Wizard” window.

- Proceed to install new drivers.

## Step 3 – Installing New USB Printer Drivers

At this stage, you should be looking at the “Hardware Update Wizard.”

1.	Select “Install from a List or Specific Location (Advanced)” and click “Next.”

2.	Insert the IJ-6000 Driver Diskette in drive A.

3.	When asked to choose the Search and Installation Options, choose “Don’t Search. I will choose the driver to install,“ and click “Next.”

4.	On the “Select the device driver you want to install for this hardware” window, click “Have Disk.”

5.	Use “Browse” button to select location of driver as A: Click “OK.”

- One of three situations can occur.

6.1 WinXP will find “A:\add_b.inf” as the best driver, and begin copying files from the diskette. You may not be notified that WinXP has found, and is using, “A:\add_b.inf”, but you will be able to observe activity on drive A.

  - If Windows XP informs it can not find a better driver for the device, either the wrong diskette has been inserted, or the old drivers have not been correctly removed. Click “Back”, check the correct diskette is inserted, and click “Next.” If the problem is not fixed call the factory for assistance.

  - If Windows XP asks you to select from a list of drivers. This situation can arise if a driver has been installed on the machine at some time for a USB device that used the same Cypress USB chip as in the Addmaster IJ-6000 printer. If you started this procedure by removing previous IJ-6000 drivers, select the “A:\add_b.inf “ entry and click “Next.” If “A:\add_b.inf” is not in the list, or if you did not begin this procedure by removing old IJ-6000 drivers, call the factory for assistance.

- Windows XP will display a copying files message. If Windows XP warns that the driver has not passed logo testing to verify its compatibility with windows XP, respond by clicking “Continue Anyway.” After files are copied and the driver is installed, the Hardware Update Wizard will inform that it has finished installing the software for “CYPRESS EZ-USB (2131Q/2131S/2135S) – EEPROM missing.” Click “Finish” to close the wizard.

After communicating with the driver Windows has installed, the printer will momentarily disconnect from the USB bus and then reconnect, asking for a second driver to be loaded.

- The Found New Hardware Wizard will inform that it is installing software for “USB Printing Support,” or for “SERIAL #34567.”

- Follow any instructions as required. Specifically, specify location “A:\” to search for a driver, and click “Continue Anyway” if challenged about an unsigned driver.

At this stage, old drivers have been deleted, the Addmaster USB driver has been replaced, the Microsoft USB printer driver has been reloaded, and the mini-driver has been optionally removed.

The next two steps are optional. Step 4 is performed to install the mini-driver if it was removed, and Step 5 tests the new Addmaster USB driver by using the mini-driver to print to the printer.

## Step 4 – Installing IJ-6000 Mini-Driver

This step is optional. It should not be performed if the mini-driver was not removed at Step 2. You may remove the mini-driver now and reinstall it if you wish. This step must be performed if the mini-driver was removed, and you intend to perform Step 5 to test the new Addmaster USB driver. However, it is safe to skip ahead to Step 6 to install the new bi-directional driver and Step 7 will test both the Addmaster USB driver and the bi-directional driver.

1.	Open the Printers and Faxes folder. Click {Start, Printers and Faxes}.

2.	Double click “Add Printer.” Windows will display the Add Printer Wizard.

3.	Choose “Local Printer” and click “Next.”

4.	Select from the list of ports. At the bottom will be an entry such as “USBnnn (Virtual printer port for USB)” entries, where nnn are numbers from 000 and 999. Typically only one entry will appear, but if previous instances of the driver were not completely removed, there may be more entries, and some trial and error may be required later to determine the appropriate selection has been made. For now, pick a USB port and click “Next,” to proceed to select the printer model

5.	“Have Disk” when prompted for printer make and model. The “Install from Disk” window will appear.

6.	Insert driver diskette into A: Use “Browse” button to select location of driver as A: Click “OK.”

- Windows will display the name of the printer type for which installation files were found. Click “Next.”

- You may get a warning to Insert Disk Labeled “Winxx Disk” and click “OK.” If so this means the A: diskette was improperly labeled. Just click “OK.” The warning is unimportant and won’t appear again.

- Select “Replace existing driver” if asked and click “Next.”

- Choose a printer name, say “IJ-6000.”

- Select “Use as default” and click “Next.”

- Select “Print test page.” and click “Next.”

- Windows will now display the “Completing the Add Printer Wizard” window. Click “Finish” to close the window.

- The process will complete automatically from here. If Windows warns “The software you are installing for this hardware has not passed Windows Logo testing to verify its compatibility with Windows XP” click “Continue Anyway.”

- Windows will copy files from the driver diskette and inform that it has sent the test page to the printer.

- Proceed to Step 5, Check Out & Test of Mini-Driver.

## Step 5 – Check Out & Test of Mini-Driver

If you skipped Step 4 above,

1.	Right click on the IJ-6000 printer icon and select “Properties.”

- Click “Print Test Page.”

If you performed step 4 above, continue here.

- If you get a message reporting an error  “Problem printing to port,” then click “OK” and proceed.

- You will be asked “Did the test page print properly?”  Regardless of whether or not it did, click “YES”. This will save time.

- If the test page actually did print properly, you are done.  If the test page failed to print properly, first check to see if both of the printer’s green lights are alight. If they are, it is waiting for a cut-form to print the test page on. Insert a cut-form and see if this fixes the problem. If it does, you are done; otherwise, continue.

- Select the “Details” tab.

- Go to the field “Print to the following port” and click the DOWN-ARROW key.

- Select another USBxxx Virtual Printer Port entry.

- Click “Apply” and then click “OK”.

- Repeat the Check Out & Test of Mini-driver process beginning at step 1.

## Step 6 – Switching to the Bi-directional Driver

This section started assuming the mini-driver was being used, and this has been tested at Step 5. Therefore, it is necessary to switch to the new bi-directional driver now. A user program is provided to perform the switch. To use run the program:

A:\ChooseIj6Driver.exe

Follow these steps:

1.	Select “Bidirectional Driver” from the list and click “Apply Choice.”

- When message “Done - Turn Printer OFF/ON to Take Effect,” appears, do so.

- If a prior bi-directional driver was installed, Windows XP will have a memory of this. Therefore, open the Device Manager and open the “Universal Serial Bus Controller” entry. If the “USB Printing Support” entry has a yellow “!” (exclamation mark) beside it, right click the entry or click “Action” from the menus, and then click “Uninstall.” Otherwise, proceed to testing step.

- Cycle printer power to activate “Found New Hardware” wizard.

- The wizard will ask you to insert the installation floppy. Insert the IJ-6000 Driver Diskette in drive A:\.

- Select “Install the Software Automatically” and click “Next”.

- Windows will access the diskette and may warn “The software you are installing for this hardware has not passed Windows Logo testing to verify its compatibility with Windows XP.” Click “Continue Anyway.”

- Wait while Windows copies files from the diskette and click “Finish” when it is done to close the wizard.

- Down in the bottom right hand corner of the screen you may catch a status message that the hardware is installed and ready to use.

- Proceed to testing.

## Step 7 – Check Out & Test of Bi-directional Driver

A test program is provided to test the connection. To use run the program:

A:\IJ6000_Test.exe

This program allows for testing the journal, validation, and status back features of the printer.

If the test program fails to print properly check these items:

- Check printer DIP switches and assure they appear in the default state.

- Make sure the IJ-6000 printer is not in its self-test mode. Turn it off and on. Wait for USB drivers to automatically reset. Try the test over again.

5.	Replacing Bi-directional Driver with Mini-Driver

Follow the step-by-step instructions in this Section when installing the software driver for Windows XP machines. Installation consists of these steps:

- Have a copy of these instructions handy for reference.

- Have the installation diskette containing the USB and printer drivers handy.

- Turn-off the IJ-6080 printer.

- Connect USB cable to computer.

- Connect USB cable to IJ-6080 printer.

## Step 1 – Preparation

Prepare for installation by collecting software and documentation:

1.	Have a copy of these instructions handy for reference.

- Have the installation diskette containing the USB and printer drivers handy.

- Turn-off the IJ-6080 printer.

- Connect USB cable to computer.

- Connect USB cable to IJ-6080 printer.

## Step 2 – Removing Old USB & Print Drivers

If you are certain that you have the latest IJ-6000 printer drivers (mini-driver and bi-directional driver) installed, skip ahead to step 4. Otherwise, continue by removing and replacing existing drivers first.

Any existing Windows USB printing support for the IJ-6000 should be removed. The IJ-6000 mini-driver can remain.

To remove old USB drivers:

1.	Connect the IJ-6000 Printer and turn it on.

- Optionally delete any IJ-6000 printers from {Start, Printers and Faxes}. Skip this step if you wish to retain the mini-driver.

- Open the Device Manager, click {Start, Control Panel, Printers and Other Hardware (switch to “Classic View”, if necessary), System, Hardware, Device Manager}.

4.	Open the “Universal serial bus controller” entry.

5.	Right click on “USB Printing Support,” or click “Action” on the top menu.

- Click the “Uninstall” option from the list menu to remove the device, and click “OK” to confirm.

- Insert “IJ-6000 Driver Diskette” into floppy drive A:

- Run program: “A:\CLEAN_XP.exe” (removes ezusb.sys, adm_ldr.sys)

- Power-off then Power-on the IJ-6000 printer.

- In the Device Manager, select the “Universal serial bus controller” entry.

- Open the “Universal serial bus controller” entry.

- Right click on the “Cypress EZ-USB(2131Q/2131S/2135S) - EEPROM missing” entry.

- Click the “Properties” option from the list menu.

- Click the “Driver” tab and then click “Update Driver.”

- Windows XP will display the “Hardware Update Wizard” window.

- Proceed to install new drivers.

## Step 3 – Installing New USB Printer Drivers

At this stage, you should be looking at the “Hardware Update Wizard.”

1.	Select “Install from a List or Specific Location (Advanced)” and click “Next.”

2.	Insert the IJ-6000 Driver Diskette in drive A.

3.	When asked to choose the Search and Installation Options, choose “Don’t Search. I will choose the driver to install,“ and click “Next.”

4.	On the “Select the device driver you want to install for this hardware” window, click “Have Disk.”

5.	Use “Browse” button to select location of driver as A: Click “OK.”

- One of three situations can occur.

6.1 WinXP will find “A:\add_b.inf” as the best driver, and begin copying files from the diskette. You may not be notified that WinXP has found, and is using, “A:\add_b.inf”, but you will be able to observe activity on drive A.

  - If Windows XP informs it can not find a better driver for the device, either the wrong diskette has been inserted, or the old drivers have not been correctly removed. Click “Back”, check the correct diskette is inserted, and click “Next.” If the problem is not fixed call the factory for assistance.

  - If Windows XP asks you to select from a list of drivers. This situation can arise if a driver has been installed on the machine at some time for a USB device that used the same Cypress USB chip as in the Addmaster IJ-6000 printer. If you started this procedure by removing previous IJ-6000 drivers, select the “A:\add_b.inf “ entry and click “Next.” If “A:\add_b.inf” is not in the list, or if you did not begin this procedure by removing old IJ-6000 drivers, call the factory for assistance.

- Windows XP will display a copying files message. If Windows XP warns that the driver has not passed logo testing to verify its compatibility with windows XP, respond by clicking “Continue Anyway.” After files are copied and the driver is installed, the Hardware Update Wizard will inform that it has finished installing the software for “CYPRESS EZ-USB (2131Q/2131S/2135S) – EEPROM missing.” Click “Finish” to close the wizard.

After communicating with the driver Windows has installed, the printer will momentarily disconnect from the USB bus and then reconnect, asking for a second driver to be loaded.

- The Found New Hardware Wizard will inform that it is installing software for “USB Printing Support,” or for “SERIAL #34567.”

- Follow any instructions as required. Specifically, specify location “A:\” to search for a driver, and click “Continue Anyway” if challenged about an unsigned driver.

At this stage, old drivers have been deleted, the Addmaster USB driver has been replaced, the mini-driver has been optionally removed, and the new bi-directional driver has been installed.

## Step 4 – Check Out & Test of Bi-directional Driver

A test program is provided to test the connection. To use, run the program:

A:\IJ6000_Test.exe

This program allows for testing the journal, validation, and status back features of the printer.

If the test program fails to print properly check these items:

- Check printer DIP switches and assure they appear in the default state.

- Make sure the IJ-6000 printer is not in its self-test mode. Turn it off and on. Wait for USB drivers to automatically reset. Try the test over again.

## Step 5 – Selecting to use the Microsoft Printer Driver / Mini-Driver Combination

This section started assuming the bi-directional driver was being used, and this has been tested at Step 4. Therefore it is necessary to switch to the Microsoft Printer Driver / mini-driver combination. A user program is provided to perform the switch. To use, run the program:

A:\ChooseIj6Driver.exe

Follow these steps:

1.	Select “Microsoft Printing (minidriver) – default” from the list and click “Apply Choice.”

2.	When message “Done - Turn Printer OFF/ON to Take Effect,” appears, do so.

3.	From this point on the process should be automatic. Windows will automatically install the Microsoft Printer Driver without any user interaction. You will see, in the bottom right hand corner of the screen, informational messages from the “Found New Hardware” wizard, that it has found device “SERIAL #34567” and then device “USB Printing Support” when it recognizes the device as a printer, and finally “Your new hardware is installed and ready to use”.

## Step 6 – Installing IJ-6000 Mini-Driver

Skip this step if the latest mini-driver is already installed.

1.	Open the Printers and Faxes folder. Click {Start, Printers and Faxes}.

2.	Click “Add Printer.” Windows will display the Add Printer Wizard.

3.	Choose “Local Printer” and click “Next.”

- Select from the list of ports. At the bottom will be an entry such as “USBnnn (Virtual printer port for USB)” entries, where nnn are numbers from 000 and 999. Typically only one entry will appear, but if previous instances of the driver were not completely removed, there may be more entries, and some trial and error may be required to determine the appropriate selection.

- “Have Disk” when prompted for printer make and model. The “Install from Disk” window will appear.

- Insert driver diskette into A: Use “Browse” button to select location of driver as A: Click “OK.”

- Windows will display the name of the printer type for which installation files were found. Click “Next.”

- You may get a warning to Insert Disk Labeled “Winxx Disk” and click “OK.” If so this means the A: diskette was improperly labeled. Just click “OK.” The warning is unimportant and won’t appear again.

- Select “Replace existing driver” if asked and click “Next.”

- Choose a printer name, say “IJ-6000.”

- Select “Use as default” and click “Next.”

- Select “Print test page.” and click “Next.”

- Windows will now display the “Completing the Add Printer Wizard” window. Click “Finish” to close the window.

- The process will complete automatically from here. If Windows warns that “The software you are installing for this hardware has not passed Windows Logo testing to verify its compatibility with Windows XP” click “Continue Anyway.”

- Windows will copy files from the driver diskette and inform that it has sent the test page to the printer.

- Proceed to Step 7, Check Out & Test of Mini-Driver.

## Step 7 – Check Out & Test of Mini-Driver

If you skipped Step 6 above,

1.	Right click on the IJ-6000 printer icon and select “Properties.”

- Click “Print Test Page.”

If you performed step 6 above, continue here.

- If you get a message reporting an error  “Problem printing to port,” then click “OK” and proceed.

- You will be asked “Did the test page print properly.”  Regardless of whether or not it did, click “YES”. This will save time.

- If the test page actually did print properly, you are done.  If the test page failed to print properly, first check to see if both of the printer’s green lights are alight. If they are, it is waiting for a cut-form to print the test page on. Insert a cut-form and see this fixes the problem. If it does, you are done; otherwise, continue.

- Select the “Details” tab.

- Go to the field “Print to the following port” and click the DOWN-ARROW key.

- Select another USBxxx Virtual Printer Port entry.

- Click “Apply” and then click “OK”.

- Repeat the Check Out & Test of Mini-Driver process at Step 1.