IJ-6080 Ink-Jet Transaction Printer

USB Bi-directional Driver Installation Guide

Addmaster Corporation

Address:	225 East Huntington Drive

Monrovia, CA 91016

Web:		www.addmaster.com

Phone:		(626) 358-2395

FAX:		(626) 358-2784

Document:	input.doc	(Source)

Revision:	1	May 7, 2002

2	April 26, 2006

Notices:	Subject to change without notice.

© Copyright 2006, Addmaster Corporation

Table of Contents

1	Introduction	3

1.1  Documentation	4

1.2  Emulations and Printer Models	4

1.3  Supplies and Ordering Information	4

2	Installation For Windows XP	5

2.1  Overview	5

2.2   Step 1 - Preparation	5

2.3   Step 2 – Removing Old USB & Print Drivers	5

2.4   Step 3 – Installing USB Communications Drivers	6

2.5   Step 4 – Installing IJ-6080 Print Driver	9

2.6   Driver Installation Verification	9

2.7   Check Out & Test	9

2.8   Test Program Source Code	10

2.9   Windows XP Diskette Contents	11

3	Document Revision Information	12

# 1	Introduction

The IJ-6080 series of Ink-Jet Transaction Printer are modern, high-reliability transaction printers designed for use in financial, POS and other applications.  The printers are available with serial, parallel, or USB interfaces.

This document provides instructions describing how to add USB print driver support for the IJ-6080 to a Windows XP based computer.

Addmaster provides two different types of drivers:

| Mini-driver | Typical Windows printer driver. Status back is not supported. Programming is optional. |
| --- | --- |
| Bi-directional | User communicates via bi-directional serial data stream with printer. Status back is supported. Programming is required. |

Note:  This document describes the Bi-directional Driver solution only.

Note:  Most users require the Mini-Driver only. Do not use the Bi-directional driver unless specified by your applications software vendor.

Only Windows XP support is given. The drivers should work fine in most Windows 2000 applications.

Figure 1.1    Addmaster IJ-6080

## 1.1  Documentation

Other available documentation concerning the Addmaster IJ-6080 series printers is:

- IJ-6080 Ink-jet Transaction Printer Specification

- IJ-6080 Ink-jet Transaction Printer User’s Guide

- IJ-6080 Ink-jet Transaction Printer Flash Programming Guide.

Contact the factory for these documents.

## 1.2  Emulations and Printer Models

There are various models of the IJ-6080 series printers. These models are all supported by the drivers provided on the diskette.

| Model | Features |
| --- | --- |
| IJ-6080 | Journal, Multi-Line Validation printing |
| IJ-6160 | Multi-Line Validation printing only |
| IJ-6080 | Journal printing only. |

For each printer, Addmaster makes available various emulations. These emulations make the printer operate and communicate similar to other existing printers. These emulations are intended for applications where existing software is designed to communicate with a particular printer.

The printer drivers provided on this diskette will work only with Addmaster standard printer emulation and not with any other emulations. Contact the factory if there are any questions.

## 1.3  Supplies and Ordering Information

Documentation and supplies may be ordered directly from the factory. Contact Addmaster at:

Addmaster Corporation

225 E. Huntington Drive

Monrovia, CA 91016

USA

Phone:	(626) 358-2395

FAX:		(626) 358-2784

Web:		www.addmaster.com

# 2	Installation For Windows XP

## 2.1  Overview

Follow the step-by-step instructions in this Section when installing the software driver for Windows XP machines. Installation consists of these steps:

- Preparing the IJ-6080 printer and the computer for installation.

- Removing any older IJ-6080 series printer drivers and any older Addmaster USB drivers.

- Installing USB printing support.

- Installing IJ-6080 series printer driver.

## 2.2   Step 1 - Preparation

Prepare for installation by collecting software and documentation:

- Have a copy of these instructions handy for reference.

- Have the installation diskette containing the USB and printer drivers handy. Copy the files to a new directory on your PC. We suggest something you can remember. Perhaps:

  - c:\Addmaster

  - c:\ij6080 drivers

- Turn-off the IJ-6080 printer.

- Connect USB cable to computer.

- Connect USB cable to IJ-6080 printer. Leave power off.

## 2.3   Step 2 – Removing Old USB & Print Drivers

If you have older IJ6080 printer drivers or USB drivers, remove them at this time. This probably does not apply to you.

## 2.4   Step 3 – Installing USB Communications Drivers

The next step involves loading the device driver responsible for communications on the USB port with the IJ-6080 printer.  This can be accomplished only with an IJ-6080 USB printer on hand and cannot be done by proxy. Each step must be followed carefully.

You should have an un-powered IJ-6080 cabled to you PC at this point (see Preparation).

- Power-on the IJ-6080 printer. Windows XP will detect the printer and display “Found New Hardware” message for “Unknown Device.”  Select “Not at this time” and hit “Next”.

- At the next screen, select “Install from a list or specific location” and click “Next.”  You have previously saved the drivers to your hard-drive. You will use these.

- At the next screen, select the box “Include this location” and then “Browse” to the directory where you have stored the drivers. In the example below, the drivers are stored in the directory “c:\ij6_cy”.   Click “Next.”

- Windows will begin installing the driver and display a copying files message. A screen will appear informing you that the driver is not signed. Click “Continue Anyway”.

- A number of files will be copied and as this is occurring Windows will inform you of the progress.

- When done with the USB communications drivers. A final prompt will tell you when this process is complete.

After the USB drivers are loaded, the printer will momentarily disconnect from the USB bus and then reconnect itself.

NOTE:

A second driver will be automatically loaded at this time.

## 2.5   Step 4 – Installing IJ-6080 Print Driver

There is no Printer Driver included or required.

The applications program connects to the device as a generic usb device with bi-directional communications. The printer is not available for general purpose Windows printing.

## 2.6   Driver Installation Verification

Not applicable.

## 2.7   Check Out & Test

A test program is provided to test the connection. It also allows for testing the journal, validation, and status back features of the printer. The program is called

ij6000_test_app.exe

and is distributed with the USB drivers. You will find this program in the directory into which you previously copied the drivers.

If the test program fails to print properly check these items:

- Check printer DIP switches and assure they appear in the default state.

- Make sure the IJ-6000 printer is not in its self-test mode. Turn it off and on. Wait for USB drivers to automatically reset. Try the test over again.

## 2.8   Test Program Source Code

The source code to the test program is provided to illustrate the coding required to utilize the IJ-6000 bidirectional driver. This code is presented in the zipped file:

ij6000_test_app_source_code.zip

Examples are included in this file that:

- Execute journal print-jobs.

- Execute validation print-jobs.

- Obtain printer real-time status information.

The code is written using Microsoft Visual C++, and may be used freely in any development project.

The files are available on Addmaster’s website

www.addmaster.com

under the “Customer Access” section.

## 2.9   Windows XP Diskette Contents

The following files are distributed for Windows XP applications.

| File Name | Use |
| --- | --- |
| Ij6_usb_bidirectional_driver_installation.pdf | Installation instructions. This file. |
| Ij6_usb1_2601.inf Ij6_usb1_2601.sys | Installation INF and driver file for: Older IJ-6080 printers. |
| Ij6_usb2_2601.inf Ij6_usb2_2601.sys | Installation INF and driver file for: Current IJ-6080 printers. Used with printers made after March 2006. |
| Ij6CyUSB2.inf Ij6CyUSB2.sys | Installation INF and driver for low-level USB communications. |
| Ij6000_test_app.exe | Test Program |

# 3	Document Revision Information

| Revision | Date | Changes / Modifications |
| --- | --- | --- |
| 1 | 10-22-2001 | Initial release in PDF format. |
| 2 | 04-26-2006 | Changes required by new release of Addmaster’s USB drivers for XP. |